Nondispersive infrared(NDIR) Gas sensor◼

C2H4 Ethylene Sensor

MST-N4L C2H4  Datasheet

Advantages& characteristics

- Quick response and accurate measurement

- Will not get poisoned

- Working with no oxygen required

- More than 5 years lifespan

- Small size structure

Application

- Environmental detection

- Gas leakage detection

- On-line monitoring

Base Specification

| ◆ Detection gas | C2H4 |
| --- | --- |
| ◆ Working principle | Non-distributed infrared (NDIR) |
| ◆ Measuring range | 0-100%LEL, 0-50%/100%vol |
| ◆ Accuracy | ±3%FS |
| ◆ Resolution | 1%LEL / 0.01%vol |
| ◆ Response time (T90) | <30s (Free diffusion) |
| ◆ Working temperature | -40℃ ~ 70℃ |
| ◆ Storage temperature | -40℃ ~ 85℃ |
| ◆ Working humidity | 0 ~ 95%RH, non-condensing |
| ◆ Preheating time | 1min |
| ◆ Digital output | TTL level |
| ◆ Analog output | 0.4 ~ 2V |
| ◆ Supply voltage | 3.0 ~ 5.5 VDC |
| ◆ Working current | Average <60mA @ 3.3VDC |
| ◆ Power consumption | 0.2W (Average value) |
| ◆ Lifespan | >5 years |
| ◆ Warranty | 1 year |
| ◆ Weight | 21g |

Pin specification

| Pin mark | Function | Description |
| --- | --- | --- |
| V+ | Positive electrode of power supply | 3.0 - 5.5 VDC |
| Gnd | Negative terminal of power supply | GND |
| Tx | Serial port transmitter | TTL TXD |
| Rx | Serial port receiver | TTL RXD |
| DA | Analog output | 0V means sensor failure 0.2V means sensor in preheating 0.4-2V corresponds to 0-100% F.S. 2-2.2V means overrange |

Size drawing (unit: mm)

V1.0 24.7.9

Ningxia MaiYa Sensor Technology Development Co., LTD

Address: Tower A, Yinchuan Zhongguancun Innovation Center, Helanshan Road, Yinchuan City, Ningxia, China750000.

Email: office@maiya-sensor.com,

Tel/Whatsapp: +86 18995278133, +86 18202760413

www.maiyachuangan.com,  www.maiya-sensor.com